Manufactured Homes

Lamar County, Georgia

Placement Guidelines

- Minimum lot size, home size, and setback requirements are found on the grid below.

- The manufactured home must meet the following standards:

  - Minimum 3/12 roof pitch

  - Minimum 16’ wide

  - Minimum landing of at least 36” by 48” at each exterior door

  - Underpinning shall be rock, block, or brick with a permanent concrete foundation

  - Previously occupied manufactured homes must be approved by the building inspector BEFORE they are moved into Lamar County

- A driveway permit and a septic tank permit must be obtained before applying for a building permit.

- The building permit will also include permit coverage for electrical, HVAC, and plumbing.